Name ____________________ Period _______

Biology

Circulation and Respiration

Blood Pressure Lab

stethoscope

Introduction

Blood pressure is the force blood exerts against the inner walls of blood vessels. Blood pressure rises and falls in a pattern corresponding to the phases of the cardiac cycle.

Contracting  ventricles (ventricular systole) squeeze blood out and

into the pulmonary arteries and aorta, this sharply increases

the pressure in these arteries.  The maximum pressure during

ventricular contraction is called the systolic pressure.

When the ventricles relax  (ventricular

diastole), pressure drops, and the lowest

pressure that remains in the arteries before

the next ventricular contraction is the

diastolic pressure.

A device called a sphygmomanometer  is used to measure blood pressure.  This vital sign is expressed as a fraction:  systolic pressure / diastolic pressure.

sphygmomanometer

sphygmomanometer

artery

cuff

pulse

bulb

Procedure

Before you begin, read all directions thoroughly. Taking another person’s blood pressure is a serious activity. A disregard for procedure can actually cause harm to your partner. Since listening is a significant part of this lab, it is also important that the classroom remain as quiet as possible to assure accurate results.

How to determine blood pressure

- Clean the earpieces of the stethoscope with an alcohol swab.

- Have the subject sit down and rest one arm on the table. Place the cuff around her/his arm slightly above the elbow so that it is snug, but not too tight. Adjust the end of the stethoscope (consult image on page 1).

- Close the valve on the pump by turning the silver knob. Pump the cuff until the gauge reads approximately 160 mm. DO NOT EXCEED 160 mm.

- To read blood pressure, you must carefully listen through the stethoscope while watching the needle on the gauge. Gently turn the silver knob to start releasing the air in the cuff. The needle should drop slowly, but not too slowly. At the first heart sound you hear, read the number at the needle.  This is the systolic pressure. Remember this number, but keep watching the gauge; when the sound disappears, read the number at the needle. This is the diastolic pressure. Record both of these numbers as a fraction.

Systolic Pressure

Diastolic Pressure

Determine blood pressure in a variety of conditions

- Record blood pressure while the subject is sitting.

- Recline subject for 5 minutes, record blood pressure after 5 minute while subject is still in a reclined position.

- Record blood pressure after subject has QUICKLY jumped up from a reclined position.

- Record blood pressure after subject has stood for 3 minutes (subject should still be standing.)

Analysis

- Collect the class data and then calculate a class average for each body condition.

- Explain the difference between systolic and diastolic in your own words.

- Atherosclerosis is a condition which the walls of large arteries change by developing fatty deposits along their surface. It is the early stage of arteriosclerosis, a condition defined by the arteries decrease in elasticity. How would atherosclerosis/arteriosclerosis impact blood pressure? Explain why!

| Group | Sitting |  | After 5 min of Reclining |  | Stand Up after reclining |  | Standing 3 minutes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |
| Average | / |  | / |  | / |  | / |  |

Name ____________________ Period _______

Blood Pressure Post Lab

Class Data